Cryo-EM sample vitrification/screening datasheet

Please fill and save in format: YYYYMMDD_CryoEM _Samplename_Surname

| Sample name | FluPol |
| --- | --- |
| Biological assembly | Influenza A virus RdRp |
| Group | Kouba |
| User name | Anatolij Filimoněnko |
| Tel. | 220 183 448 |
| Email | anatolij.filimonenko@uochb.cas.cz |
| Date (YYYYMMDD) | 20210911 |
| Negative stain experiment date(s) (YYYYMMDD) | 20211105 |
| Molecular weight of full biological assembly (kDa) | 351 kDa |
| Oligomeric state (monomer, dimer) | heterotrimer |
| Concentration of full biological assembly (uM, minimum cca 1 uM) | 1,2 uM |
| Storage/freezing conditions (4°/-20°/-80°) | -80°/ LN flash freeze |
| 1x sample buffer | 20 mM HEPES pH 7.5, 150 mM NaCl, 5 mM MgCl2, 1 mM TCEP |
| Supplied sample buffer aliquot concertation (1x, 10x) (minimum 1mL) | 10x |
| Additional information if available: |  |
| PDB code of known or homologous structure | 6RR7 |

Notes: Pure protein on SDS-PAGE

Sample directly from SEC run

Negative stain image:

20211105

Cryo-EM sample vitrification/screening datasheet results:

To be filled by the cryo-EM facility

| Vitrification: Glow discharge; 2x20s 25mA 0.3mBar. Temperature/humidity; 4°/100%. Blotting force/time; -5/2s. Grid type; Q gold 300 mesh R1.3/1.2. Final sample conc. (full biological assembly in uM); 0.8uM. Sample application; both sides 1.5+1.5ul. Grid table: Grid position; Status. 20211125_I1; screened. 20211125_I2; stored. 20211125_I3; stored. 20211125_I4; stored. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Vitrification:

| Glow discharge | 2x20s 25mA 0.3mBar |
| --- | --- |
| Temperature/humidity | 4°/100% |
| Blotting force/time | -5/2s |
| Grid type | Q gold 300 mesh R2/1 |
| Final sample conc. (full biological assembly in uM) |  |
| Sample application | both sides 1.5+1.5ul |

Grid table:

| Grid position | Status |
| --- | --- |
| 20211125_II1 | stored |
| 20211125_II2 | stored |
| 20211125_II3 | stored |
| 20211125_II4 | screened |

Screening:

Sample name: YYYYMMDD_gridplace_CryoEM_Samplename_Surname

Atlas image:

Name: 20211125_I1_CryoEM_ Samplename_Surname _atlas.jpg

Representative images:

Square1

Name: 20211125_I1_CryoEM_ Samplename_Surname _sq1.jpg

Note: Small medium thick ice square

Hole1

Name: 20211125_I1_CryoEM_ Samplename_Surname _sq1_hole1.jpg

Note: Ice homogeneous within the hole

| Pixel size (A/pixel) | 2.5 |
| --- | --- |
| Defocus (um) | -3.0 |
| Total exposure e/A2 | 42 |
| Operator | Kouba |
| Microscope/camera | FEI_Spectra/Gatan-Orius |

Particles1:

Name:20211125_I1_ Samplename_Surname _sq1_hole1_par1.jpg

Note: Even distribution of particles throughout the hole, decent background

Square2

Name: 20211125_I1_CryoEM_ Samplename_Surname _sq2.jpg

Note: Big thin ice square

Hole2

Name: 20211125_I1_CryoEM_ Samplename_Surname _sq2_hole1.jpg

Note: Visible gradient of ice within the hole, very thin middle

| Pixel size (A/pixel) | 2.5 |
| --- | --- |
| Defocus (um) | -3.0 |
| Total exposure e/A2 | 42 |
| Operator | Kouba |
| Microscope/camera | FEI_Spectra/Gatan-Orius |

Particles2:

Name: 20211125_I1_CryoEM_ Samplename_Surname _sq2_hole1_par1.jpg

Note: Particles only at edges, central part denatured

Summary:

Gradient of ice within the grid, large thin ice squares, medium size squares with medium thick ice, small square with very thick ice

No contaminations

Usable for high-res data collection

Use only smaller medium thick squares where particles are not denatured, or collect data on large thin squares but only around the very hole edges